jAligner4GCxGC

I. Installation (package-based structure)

Before using the package, several applications/tools must be installed:

- Visual Studio Code (VS Code); https://code.visualstudio.com/download.

- Julia Programming Language (version 1.10.11); open Command Prompt > Type or copy in juliaup add 1.10.11 > Press Enter.

- Julia Extension in VS Code > Extensions > Search for “julia” > Press Install.

Then, the package should be loaded as follows:

- Go to the package repository at https://github.com/r3bryk/jAligner4GCxGC and download run_me.jl. Alternatively, open run_me.jl and copy the code to an empty VS Code notebook file.

- Open VS Code > Press “Ctrl + Shift + P” > Type or copy in “Julia: Start REPL” > Click on Julia: Start REPL in the drop-down menu > Wait until you see julia> in the VS Code terminal; this message will appear after any operation is completed and Julia is ready to proceed.

- The package can be installed from run_me.jl file as follows:

  - Enable using packages by highlighting using Pkg in line 10 and pressing “Ctrl + Enter”.

  - Get the package by highlighting line 14 and pressing “Ctrl + Enter”. This procedure will take some time. When done, you will see a message julia> in the terminal.

- Alternatively, copy the two lines

using Pkg

Pkg.add(url="https://github.com/r3bryk/jAligner4GCxGC")

Paste into the terminal and press “Enter”.

- Open run_me.jl in VS Code.

II. How to use the script

To use the script, the following steps should be executed:

- All the input files, that are to be aligned, must be copied to an empty working folder, the path to which must be specified in run_me.jl at line 36 as path2files = "D:\\Projects\\Test", where "D:\\Projects\\Test” is the path to your working folder. Load input files by highlighting line 33-39 and pressing “Ctrl + Enter”.

- Convert TXT files to CSV format by highlighting line 44 (jAligner4GCxGC.convert_txt2csv(path2files)) and pressing “Ctrl + Enter”.

- Filter out rows for which "Classifications" column contains 'Bleed', 'Toluene', or 'DCM' by highlighting line 49 (jAligner4GCxGC.filter_classifications(path2files)) and pressing “Ctrl + Enter”.

- Filter out rows where "Name" contains unwanted values, e.g., siloxanes and other bleed:

  - Specify keywords (unwanted compound names or parts of names) to filter out in the keywords array. The names should be in square brackets, double quoted, and separated by commas, e.g., keywords = ["1H-Tetrazol-5-amine", "Silic", "TMS"]. Then, load the keywords array by highlighting lines 24-65 and pressing “Ctrl + Enter”.

  - Filter out rows where "Name" contains unwanted values by highlighting line 68 (jAligner4GCxGC.filter_names(path2files, keywords)) and pressing “Ctrl + Enter”.

- Filter out rows where "Name" contains, e.g., compounds with spectra similar to toluene (m/z 91 or 92), if their spectra do not contain other m/z values (m/z’s > 91/92) such as molecular ions:

  - Specify filters as a list of tuples (keyword, spectrum_number (specific integer m/z in spectrum)), e.g., filters = [("(Isopropoxymethy)lbenzene", 107), ("2-Butanol, 3-benzyloxy-", 135), ("Bibenzyl", 182)]. Then, load the filters by highlighting lines 73-200 and pressing “Ctrl + Enter”.

  - Filter out rows by highlighting line 204 (jAligner4GCxGC.filter_names_if_missing_mz(path2files, filters)) and pressing “Ctrl + Enter”.

- Filter out rows where "Name" contains "Peak" (a marker for unknown feature) if their spectra contain bleed m/z values such as 73 or 207:

  - Specify bleed filters as a list of tuples (keyword, spectrum_number (specific integer m/z in spectrum)), e.g., bleed_filters = [("Peak", 73), ("Peak", 147), "Peak", 207)]. Then, read the bleed filters by highlighting lines 209-216 and pressing “Ctrl + Enter”.

  - Filter out rows by highlighting line 219 (jAligner4GCxGC.filter_peak_names_with_bleed_mz(path2files, bleed_filters)) and pressing “Ctrl + Enter”.

- If needed, reset masses classified/filtered by jAligner4GCxGC.filter_signal_external function (previous classification; see below) by unmuting line 225 (jAligner4GCxGC.reset_filters(path2files)) by highlighting it and pressing “Ctrl + /” and run it by pressing “Ctrl + Enter”.

- Remove m/z:intensity pairs with intensity values containing '0.' from spectra by highlighting line 231 (jAligner4GCxGC.remove_zero_intensity_pairs(path2files)) and pressing “Ctrl + Enter”.

- Remove m/z:intensity pairs with intensity values ≤ 3% of the highest intensity value by highlighting line 236 (jAligner4GCxGC.remove_low_intensity_pairs(path2files)) and pressing “Ctrl + Enter”.

- Classify features as "Bleed" by base masses or filter out features by base masses or mass/intensity combinations:

  - Specify in lines 242-253 the input parameters, such as:

    - Base mass(es) (base = [73, 147, 207]).

    - Other signals that need to be present (mz).

    - Relative intensity of those signals with 1 = 100% (int).

    - 1st and 2nd dimension retention ranges mass(es) can occur in (tr1_range, tr2_range).

    - m/z tolerance (mz_tol = 0.1).

    - Load the parameters by highlighting lines 242-250 and pressing “Ctrl + Enter”.

  - Classify features by highlighting line 253 (jAligner4GCxGC.filter_signal_external(path2files, filtType, base, mz, int, mz_tol, tr1_range, tr2_range)) and pressing “Ctrl + Enter”.

- Classify features as " Toluene" by base masses or filter out features by base masses or mass/intensity combinations:

  - Specify in lines 258-267 the input parameters, such as:

    - Base mass(es) ( base = [91]).

    - Other signals that need to be present (mz = [92]).

    - Relative intensity of those signals with 1 = 100% (int = [0.5]).

    - 1st and 2nd dimension retention ranges mass(es) can occur in (tr1_range, tr2_range).

    - m/z tolerance (mz_tol = 0.1).

    - Load the parameters by highlighting lines 258-264 and pressing “Ctrl + Enter”.

  - Classify features by highlighting line 267 (jAligner4GCxGC.filter_signal_external(path2files, filtType, base, mz, int, mz_tol, tr1_range, tr2_range)) and pressing “Ctrl + Enter”.

- Classify features as "DCM" by base masses or filter out features by base masses or mass/intensity combinations:

  - Specify in lines 272-281 the input parameters, such as:

    - Base mass(es) ( base = [49]).

    - Other signals that need to be present (mz = [84]).

    - Relative intensity of those signals with 1 = 100% (int = [0.5]).

    - 1st and 2nd dimension retention ranges mass(es) can occur in (tr1_range, tr2_range).

    - m/z tolerance (mz_tol = 0.1).

    - Load the parameters by highlighting lines 272-278 and pressing “Ctrl + Enter”.

  - Classify features by highlighting line 281 (jAligner4GCxGC.filter_signal_external(path2files, filtType, base, mz, int, mz_tol, tr1_range, tr2_range)) and pressing “Ctrl + Enter”.

- Classify features as "Alkane" by base masses or filter out features by base masses or mass/intensity combinations:

  - Specify in lines 286-295 the input parameters, such as:

    - Base mass(es) ( base = [57]).

    - Other signals that need to be present (mz = [71, 85]).

    - Relative intensity of those signals with 1 = 100% (int = [0.3, 0.2]).

    - 1st and 2nd dimension retention ranges mass(es) can occur in (tr1_range, tr2_range).

    - m/z tolerance (mz_tol = 0.1).

    - Load the parameters by highlighting lines 286-292 and pressing “Ctrl + Enter”.

  - Classify features by highlighting line 295 (jAligner4GCxGC.filter_signal_external(path2files, filtType, base, mz, int, mz_tol, tr1_range, tr2_range)) and pressing “Ctrl + Enter”.

- Classify features as "Acid" by base masses or filter out features by base masses or mass/intensity combinations:

  - Specify in lines 300-309 the input parameters, such as:

    - Base mass(es) (base = [55, 60, 73]).

    - Other signals that need to be present (mz = [60, 73]).

    - Relative intensity of those signals with 1 = 100% (int = [0.5, 1]).

    - 1st and 2nd dimension retention ranges mass(es) can occur in (tr1_range, tr2_range).

    - m/z tolerance (mz_tol = 0.1).

    - Load the parameters by highlighting lines 300-306 and pressing “Ctrl + Enter”.

  - Classify features by highlighting line 309 (jAligner4GCxGC.filter_signal_external(path2files, filtType, base, mz, int, mz_tol, tr1_range, tr2_range)) and pressing “Ctrl + Enter”.

- If needed (not recommended), merge internal drifting features (features of the same chemical origin, e.g., potential main peak and tailing peak):

  - Specify in lines 316-319 the input parameters, such as:

    - Retention deviation right and left of the peak in the 1st dimension (tr1_dev = [3, 5]).

    - Retention deviation up and down of the peak in 2nd dimension (tr2_dev = [0.05, 0.1]).

    - Spectral similarity threshold for merging (sim_thresh = 0.8).

    - m/z tolerance (mz_tol = 0.1).

    - Load the parameters by highlighting lines 316-319 and pressing “Ctrl + Enter”.

  - Merge internal drifting features by highlighting line 322 (jAligner4GCxGC.internal_feature_merging(path2files, tr1_dev, tr2_dev, sim_thresh, mz_tol)) and pressing “Ctrl + Enter”.

- Align samples:

  - Specify in lines 329-340 the input parameters, such as:

    - Area value above which the highA RT threshold is used (h_thresh = 10000000).

    - 1st dimension RT tolerance for low intensity peaks (rt_tol_lowA = 7).

    - 1st dimension RT tolerance for high intensity peaks (rt_tol_highA = 11).

    - 2nd dimension RT tolerance for low intensity peaks (rt2_tol_lowA = 0.1).

    - 2nd dimension RT tolerance for high intensity peaks (rt2_tol_highA = 0.15).

    - Spectral similarity threshold for alignment (sim_thresh = 0.8).

    - m/z tolerance (mz_tol = 0.1).

    - Method how the consensus spectrum is setup; "height" weights the spectrum based on the base peak height, "equal" assumes equal contribution (overviewMerging = "height").

    - Weights used for score calculation [similarity, library probability, difference RI (library vs. experimental)] (weights = [1, 1, 1]).

    - Number of parts the 1st RT range is split into for alignment (tr1_parts = 5).

    - Load the parameters by highlighting lines 329-340 and pressing “Ctrl + Enter”.

  - Align samples by highlighting line 343-344 (jAligner4GCxGC.feature_align_external(path2files, rt2_tol_lowA, rt2_tol_highA, rt_tol_lowA, h_thresh, rt_tol_highA, sim_thresh, mz_tol, overviewMerging, weights, tr1_parts; numfrags, similarity_method)) and pressing “Ctrl + Enter”.

- Filter out after alignment rows for which "Name" contains, e.g., compounds with spectra similar to toluene’s (m/z 91 or 92), if their spectra do not contain other m/z values (m/z’s > 91/92) such as molecular ions by highlighting line 350 (jAligner4GCxGC.filter_names_mz_postalign(path2files, filters)) and pressing “Ctrl + Enter”.

- Merge features with the same name and base m/z if their ΔRI < 7.0 by highlighting line 355 (jAligner4GCxGC.merge_postalign(path2files, 7.0)) and pressing “Ctrl + Enter”.

- Retrieve InChIKey, SMILES, CAS#, DTXSID, Uses, and Use Classification values from PubChem by highlighting line 360 (jAligner4GCxGC.PubChemRetriever(path2files::String)) and pressing “Ctrl + Enter”.

- Retrieve functional use data from, e.g., EPA FUse, NORMAN dust trial, and in-house harmonized use databases, or any other database(s) at hand:

  - Specify in lines 366-375 paths to the working file and the 3 files with classification data as:

    - path_file = "D:\\Projects\\Test\\Aligned_File.csv". Required columns: Name, InChIKey_Consensus, and CAS_PubChem.

    - path_harm = "D:\\Projects\\Test\\Databases\\Classes_Harmonized.txt". Required columns: Name, InChIKey, Class, and CAS.

    - path_fuse = "D:\\Projects\\Test\\Databases\\FUse_Database.txt". Required columns: Name, InChIKey, harmonized_function, and CAS.

    - path_norm = "D:\\Projects\\Test\\Databases\\NORMAN_Dust_Compounds.txt". Required columns: Name, InChIKey, Class, and CAS.

NB! The script can handle several other column name options like Feature name, Metabolite name, inchikey, CASRN, CAS#, etc. without crashing.

    - Load the files by highlighting lines 366-375 and pressing “Ctrl + Enter”.

  - Retrieve functional use data by highlighting line 379 (jAligner4GCxGC.ClassFinder(path_file, path_harm, path_fuse, path_norm)) and pressing “Ctrl + Enter”.

- Search library/database:

  - Specify library/database and aligned files as:

    - pathDB = "D:\\Projects\\Test\\Dust_Database.xlsx" in line 385. Required columns: "Spectrum", "#", "Name", "RI", "CAS Number", "InChIKey", "Formula".

    - path_file = "D:\\Projects\\Test\\Aligned_File.csv" in line 388. Required columns: "Name" and/or "All_Names", "Spectrum_BestMatch" and/or "Spectrum_Consensus", "BaseMass" and/or "All_BaseMass", "Origin" and/or "All_Origin", "Class" and/or "All_Class", "MinRIlib", "MinRI", "MaxRIlib", "MaxRI", "Nr".

    - Load the files by highlighting lines 385-389 and pressing “Ctrl + Enter”.

  - Specify in lines 392-397 the input parameters, such as:

    - RI windows for semi-standard non-polar RI, standard non-polar RI, standard polar RI, and estimated RI (addRIwin = [30, 50, 50, 100], respectively). If you do not have any of the values for a particular compound, leave it blank; all the RI values can be left blank, but then the match will be based only on spectral similarity.

    - Which of the spectra should be matched: "BM" = best match, "CS" = consensus (specID = "CS"), m/z tolerance (mz_tol = 0.1).

    - Minimum number of highest intensity fragments used from each spectrum; default = 15 (numfrags = 50).

    - Which features should be used for library matching: "all" uses every entry, a vector of numbers [1, 2, 6] uses only those indices, and collect(1:100) uses only indices/entries from 1 to 100 (index = "all").

    - Load the parameters by highlighting lines 392-397 and pressing “Ctrl + Enter”.

  - Run library search by highlighting line 400 (jAligner4GCxGC.librarySearch(pathDB, pathFile, addRIwin, specID, mz_tol, numfrags, index, similarity_method)) and pressing “Ctrl + Enter”.

- Create head-to-tail graphs for library vs. experimental (user) spectra:

  - Specify the input parameters, such as:

    - Path to file with library search results, e.g., pathFile = "D:\\Projects\\Test\\Aligned_File_LibSearch_CS.csv" in line 406.

    - Which features should be used for library matching in line 410: "all" uses every entry, a vector of numbers [1, 2, 6] uses only those indices, and collect(1:100) uses only indices/entries from 1 to 100 (index = "all").

    - Load the parameters by highlighting lines 406-410 and pressing “Ctrl + Enter”.

  - Run visualization by highlighting line 411 (jAligner4GCxGC.libraryVisualization(pathFile, index)) and pressing “Ctrl + Enter”.

- If needed, resort columns in a specific order and delete specified columns of low importance or QC columns from the aligned table:

  - Specify in lines 417-441 the input parameters, such as:

    - Path to file to be processed, e.g., fileColDrop = "D:\\Projects\\Test\\Aligned_File.csv" in line 417.

    - Metadata column order, e.g., meta_order = ["Nr","MinRt1D","MaxRt1D","AveRt1D","MinRt2D","MaxRt2D","AveRt2D","Spectrum_BestMatch"].

    - Names of the columns to drop, e.g., col_names = ["MinRt1D", "MaxRt1D", "MinRt2D", "MaxRt2D"].

    - Load the parameters by highlighting lines 417-441 and pressing “Ctrl + Enter”.

  - Run the function by highlighting line 444 (jAligner4GCxGC.resort_delete_cols(path2files, fileColDrop, col_names, meta_order)) and pressing “Ctrl + Enter”.

Notes and recommendations

It is recommended to use all the steps, except for steps 7 and 15, as some of the functions use the files or columns created by functions in previous steps by their specific filenames or column names, and/or extensions. Hence, if one of those functions was not used and the respective file was not created, the script will crash and return error. Alternatively, in case a function should be skipped, the file(s) can be renamed manually and then used by the respective function. For input filenames and suffices, see the code of the function in question in src/jAligner4GCxGC.jl, the part with input file specification, e.g., if occursin("_Mrgd.csv", f).

The following functions can be skipped with no harm to script execution:

- jAligner4GCxGC.filter_classifications(path2files)

- jAligner4GCxGC.filter_names(path2files, keywords)

- jAligner4GCxGC.filter_names_if_missing_mz(path2files, filters)

- jAligner4GCxGC.filter_peak_names_with_bleed_mz(path2files, bleed_filters)

- jAligner4GCxGC.reset_filters(path2files)

- jAligner4GCxGC.remove_zero_intensity_pairs(path2files)

- jAligner4GCxGC.remove_low_intensity_pairs(path2files)

- jAligner4GCxGC.internal_feature_merging(path2files, tr1_dev, tr2_dev, sim_thresh, mz_tol)

The script takes TXT (tab-delimited) files, such as LECO ChromaTOF result files, as input.

The input files must contain at least the following columns to be processed:

"Name", "R.T. (s)", "Area", "Spectrum", "Formula", "CAS", "InChIKey", "Retention Index", "Lib. RI", "Height", "Similarity", "Base Mass", "Actual Masses", "FWHH (s)", "Quant Masses", "Probability", "Expected Ion m/z", "Observed Ion m/z", "Classifications"

NB! "Spectrum" values must be in LECO ChromaTOF result file format, i.e. 37:1070 38:1930 39:4500 40:3720 41:4570 49:30 50:130 51:90 52:220 62:50 63:120 64:80 67:9999 68:460.

Example of the input file:

The library/database file must contain at least the following columns to be processed:

"#", "Name", "InChIKey", "CAS Number", "Formula", "Spectrum", “RI”, “RI StdNP”, “RI Est”,

where “RI” is semi-standard non-polar RI, “RI StdNP” is standard non-polar RI, “RI StdPolar” is standard polar RI, and “RI Est” is estimated RI. If you do not have any of the RI values for a particular compound, leave the respective column cell blank; all the RI values can be left blank, but then the match will be based only on spectral similarity.

NB! "Spectrum" values must be in LECO ChromaTOF result file format, i.e. 37:1070 38:1930 39:4500 40:3720 41:4570 49:30 50:130 51:90 52:220 62:50 63:120 64:80 67:9999 68:460.

Example of the library/database file:

III. Main functions & what they do

All the functions form a comprehensive workflow for processing, analyzing, and summarizing mass spectrometry data.

### Data Import and Cleaning

- report_import_external: Imports and cleans data from multiple .csv files.

- convert_txt2csv: Converts .txt files to .csv format for further processing.

- reset_filters: Restores original files by removing filtered and merged files.

- inchiExtraction: Cleans up InChIKey values.

### Spectrum Processing

- getSpec: Splits a spectrum string and turns it into a 2D array.

- master_mass_gen: Generates a master list of m/z values and their corresponding intensities from two sets of input data.

- mergeSpec: Merges spectra by summing the intensities of identical m/z values.

### Feature Selection and Grouping

- select_candidates_rt1_: Selects candidate features for RT1 based on a tolerance.

- select_candidates_rt2_: Selects candidate features for RT2 based on a tolerance.

- select_candidates_dot_: Selects candidate features based on spectra similarity.

- group_candidates_external: Groups candidates and calculates various statistics for each group.

- feature_align_fraction: Aligns RT1 fractions (parts) by selecting candidate features based on retention times, similarity, and other criteria.

- feature_align_external: Performs a comprehensive alignment of features from multiple .csv files.

### Feature Merging, Filtration, and Classification

- internal_feature_merging: Merges features of the same chemical origin within a sample.

- filter_signal_external: Classifies features based on their base mass and retention time (RT1/RT2) region.

- filter_classifications: Filters out rows where the 'Classifications' column contains unwanted values.

- filter_names: Filters out rows where the 'Name' column contains unwanted values.

- filter_names_if_missing_mz: Filters out rows where the 'Name' column contains specific keywords and the spectra do not contain specified m/z values.

- filter_names_mz_postalign: Filters out rows after alignment based on specific name and m/z conditions.

- filter_peak_names_with_bleed_mz: Filters out rows based on presence of “Peak” (marker for unknown compound) in name and m/z conditions.

- merge_postalign: Merges features with the same name, base mass, and dRI < threshold after alignment.

### Similarity Calculation

- findMaxSim: Retrieves the feature with the maximum similarity score from a list of similarity scores, probabilities, and retention index differences.

### Data Visualization

- libraryVisualization: Creates head-to-tail graphs for user vs. library spectra.

### Data Retrieval and Enrichment

- PubChemRetriever: Retrieves InChIKey, SMILES, and CAS# values from PubChem based on existing “InChIKey” and "Name" (if InChIKey does not exist) column values.

- ClassFinder: Retrieves functional use data from EPA FUse, NORMAN dust trial, and in-house harmonized use databases.

### Library Search

- librarySearch: Searches a library/database post-alignment to potentially identify unknowns and verify previous identifications.

### Utility Functions

- mergeToLeft: Merges RI values (“RI”, “RI StdNP”, “RI StdPolar”, “RI Est”) to the leftmost non-missing value and pairs them with corresponding RI tolerance values.

- remove_zero_intensity_pairs: Removes m/z:intensity pairs with intensity values containing '0.' from spectra.

- remove_low_intensity_pairs: Removes m/z:intensity pairs with intensity values <= 3% of the highest intensity values for each spectrum.

- escape_regex: Escapes special regex characters in a string.

### Additional Functions

- calcTotalArea: Calculates the total area for each entry based on the provided area, quant mass, and spectrum.

- extractRTs: Extracts RT1 and RT2 values from the 'R.T. (s)' column in the DataFrame.

- finalOverviewTable: Creates an aligned overview table from various input tables, merging spectra and calculating detection frequencies and other statistics.

- resort_delete_cols: Resort columns in a specific order and deletes specified columns.

### Overall Script Workflow

- Data Import and Cleaning:

  - The workflow begins with importing and cleaning the data using report_import_external, convert_txt2csv, reset_filters, and inchiExtraction. This ensures that the data is in a consistent and usable format.

- Spectrum Processing:

  - Spectra are processed using getSpec, master_mass_gen, and mergeSpec to prepare them for feature selection and comparison.

- Feature Selection and Grouping:

  - Candidate features are selected based on retention times and spectra similarity using select_candidates_rt1_, select_candidates_rt2_, and select_candidates_dot_. These candidates are then grouped and aligned using group_candidates_external, feature_align_fraction, and feature_align_external.

- Feature Merging and Classification:

  - Features are merged, filtered, and classified using internal_feature_merging, filter_signal_external, filter_classifications, filter_names, filter_names_if_missing_mz, filter_peak_names_with_bleed_mz, filter_names_mz_postalign, and merge_postalign. This step ensures that the data is accurate and relevant.

- Similarity Calculation:

  - The findMaxSim function calculates similarity scores to identify the best matches among the features.

- Data Visualization:

  - The libraryVisualization function creates visual representations of the data (head-to-tail graphs for user vs. library spectra), providing a clear comparison between user and library spectra.

- Data Retrieval and Enrichment:

  - Additional data is retrieved from external databases using PubChemRetriever and ClassFinder, enriching the dataset with chemical and functional information.

- Library Search:

  - The librarySearch function searches a library/database to identify unknowns and verify previous identifications, enhancing the dataset with verified identifications.

- Utility Functions:

  - Utility functions like mergeToLeft, remove_zero_intensity_pairs, remove_low_intensity_pairs, and escape_regex provide additional support for data processing and cleaning.

- Additional Functions:

  - Functions like calcTotalArea, extractRTs, and finalOverviewTable provide essential calculations and summaries, ensuring that the data is comprehensive and well-organized.

### 1. report_import_external

This function imports, cleans, and aligns data from multiple .csv files in a specified directory, ensuring consistency and handling missing values appropriately. It processes the data to create a comprehensive DataFrame with aligned and cleaned data.

#### Input and Output

- Input Files; the function processes multiple .csv files containing various attributes of the data.

  - Required Columns in Input Files:

  - F_ID: Unique identifier for each entry.

  - Name: Name of the entry.

  - R.T. (s): Retention time in seconds.

  - Area: Area under the curve.

  - Spectrum: Spectrum data.

  - Formula: Chemical formula.

  - CAS: CAS number.

  - InChIKey: InChIKey values.

  - Retention Index: Retention index.

  - Lib. RI: Library retention index.

  - Height: Peak height.

  - Similarity: Similarity score.

  - Base Mass: Base mass.

  - Actual Masses: Actual masses.

  - FWHH (s): Full width at half height in seconds.

  - Quant Masses: Quantitative masses.

  - Probability: Probability values.

  - Expected Ion m/z: Expected ion mass-to-charge ratio.

  - Observed Ion m/z: Observed ion mass-to-charge ratio.

  - Classifications: Class information.

  - Origin: Chemical origin information; created by the filter_signal_external function.

- Output:

  - Rep: A DataFrame containing the aligned and cleaned data.

  - namesT: A list of file names that were processed.

#### Function Steps

- Initialization:

  - Prints a message to ensure the directory contains only relevant .csv files.

  - Reads the directory contents and initializes an empty DataFrame Rep and an empty list namesT.

- Load Files:

  - Iterates through each file in the directory.

  - Skips files that do not meet specific criteria (e.g., not .csv files, or containing certain keywords like "Aligned", "original", etc.).

  - Reads the valid .csv files into a DataFrame df, selecting specific columns.

  - Adds a new column F_ID to df and appends it to Rep.

  - Updates namesT with the file names.

- Data Cleaning:

  - Removes rows with missing 'Area' entries.

  - Converts retention times using the extractRTs function.

  - Replaces missing values in 'CAS', 'InChIKey', and 'Formula' columns with "NA".

  - Fixes missing 'Similarity' scores by setting them to -1.

  - Converts 'Peak #' to 'Peak_rt1_rt2' format based on retention times.

  - Cleans up 'InChIKey' values using the inchiExtraction function and creates truncated InChI keys.

  - Ensures 'Lib. RI' values are numbers only and calculates the difference between 'Retention Index' and 'Lib. RI' (dRI).

- Fix General Missing Values:

  - Creates a new 'Probability' column with the correct type and replaces missing values with NaN.

  - Replaces missing values in 'Expected Ion m/z' and 'Observed Ion m/z' with NaN.

  - Handles missing 'Classifications' by filling them with "NA".

- Area Re-calculation:

  - Calculates the total area (totArea).

- Convert to General Format:

  - Converts the DataFrame Rep to a general format with specific columns: F_ID, Name, Int (total area), Rt (1st dimension RT), Rt_2D (2nd dimension RT), Spec (Spectrum), Form (Formula), CAS, Inchi (InChIKey), InchiTrunc (truncated InChIKey), RI (Retention Index), RIlib (Library RI), Height, Sim (Similarity), dRI (difference between Retention Index and Library RI), FWHH (Full Width at Half Height), libProb (Probability), ExpIon (Expected Ion m/z), ObsIon (Observed Ion m/z), BaseMass, ActualMass, QuantMass, Class (Classifications), Origin.

- Return:

  - Returns the processed DataFrame Rep and the list of file names namesT.

### 2. convert_txt2csv

This function converts .txt files to .csv format , ensuring that all relevant files in the specified directory are processed and converted appropriately.

#### Input and Output

- Input Files: The function processes multiple .txt files in the specified directory. The function does not require specific columns.

- Output:

  - The function generates .csv files corresponding to each .txt file in the directory.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

- Conversion:

- Iterates through each file in the directory.

- Checks if the files has a .txt extension.

- Converts .txt files to .csv format using CSV.read and CSV.write.

- Writes the converted data to a new .csv file with the same name as the original .txt file.

### 3. reset_filters

This function resets previous filters (classifications) made by the filter_signal_external function by restoring original files, ensuring that any filtered or merged files are removed, and the original files are reinstated.

#### Input and Output

- Input Files: The function processes multiple .csv files in the specified directory. The function does not require specific columns; it operates based on file names and extensions.

- Output:

  - The function restores the original .csv files by removing any filtered or merged versions.

#### Function Steps

- Initialization:

  - Reads the directory contents and finds files with "original" in their names.

- Resetting Filters:

  - For each original file, removes the corresponding filtered (if it exists) and merged files (if it exists).

  - Renames the original file back to its original name.

### 4. inchiExtraction

This function extracts and cleans up InChIKey values, ensuring that they are in a consistent and usable format.

#### Input and Output

- Input: The function processes an array of InChIKey strings. The function does not require specific columns; it operates on the provided array of InChIKey strings.

- Output:

  - The function returns a modified array of InChIKey strings with cleaned and standardized values.

#### Function Steps

- Loop through InChI Strings:

  - Iterates over each element in the inchi array.

  - Replaces "NA" with "NoID".

  - Extracts relevant parts from strings containing "href=".

  - Prints a message for unknown formats and leaves the element unchanged.

- Return:

  - Returns the modified inchi array.

### 5. getSpec

This function splits a spectrum string and converts it into a 2D array, ensuring that the data is properly formatted and parsed into numerical values.

#### Input and Output

- Input: The function processes a spectrum string. The function does not require specific columns; it operates on the provided spectrum string.

- Output:

  - The function returns a 2D array of spectrum values parsed as Float64.

#### Function Steps

- Split and Parse:

  - Checks if the spectrum string has a trailing space and removes it if present.

  - Splits the string by spaces and colons.

  - Parses the split strings into a 2D array of Float64 values.

  - Reshapes the parsed values into a 2D array.

- Return:

- Returns the 2D array of spectrum values.

### 6. master_mass_gen

This function generates a master list of m/z values and their corresponding intensities from two sets of input data, ensuring that the data is properly aligned and merged based on a specified mass tolerance.

#### Input and Output

- Input: The function processes two sets of m/z values and their corresponding intensities.

- Required Columns in Input Files:

- mz_ref: Reference m/z values.

- int_ref: Reference intensities.

- mz_user: User m/z values.

- int_user: User intensities.

- mass_tol: Mass tolerance for matching m/z values.

- Output:

  - The function returns a 2D array master_vect_f containing the master list of m/z values and their corresponding intensities.

#### Function Steps

- Concatenate m/z Values:

  - Combines the reference m/z values (mz_ref) and user m/z values (mz_user) into a single array mz_values.

- Initialize Master Vector:

  - Creates a master_vect array with the same length as mz_values, initialized with zeros.

- First Loop: Average m/z Values:

  - For each m/z value in mz_values, it finds all values within half of the mass tolerance.

  - Calculates the mean of these values and assigns it to the first column of master_vect.

- Second Loop: Assign Intensities:

  - For each non-zero m/z value in master_vect, it finds the corresponding reference and user intensities within the mass tolerance.

  - Assigns the maximum reference and user intensities to the second and third columns of master_vect.

- Filter Non-Zero m/z Values:

  - Filters out rows in master_vect where the m/z value is zero.

- Return:

- Returns the filtered master_vect_f array containing the master list of m/z values and their corresponding intensities.

### 7. mergeSpec

This function merges spectra by summing the intensities of identical m/z values, ensuring that the resulting spectrum is properly aggregated.

#### Input and Output

- Input: The function processes a 2D array of spectrum values. The function does not require specific columns; it operates on the provided 2D array of spectrum values.

- Output:

  - The function returns a 2D array sp_m containing the merged spectrum with summed intensities.

#### Function Steps

- Initialization:

  - Initializes an array sp_m with unique m/z values.

- Sum Intensities:

  - For each unique m/z value, it sums the intensities of all matching m/z values in the input spectrum.

- Return:

  - Returns the merged spectrum sp_m  with summed intensities, converted to integers.

### 8. select_candidates_rt1_

This function selects candidate features for RT1 based on a specified retention time tolerance, ensuring that the selected features fall within the defined bounds.

#### Input and Output

- Input: The function processes a DataFrame of features and an index of the feature of interest.

- Required Columns in Input Files:

- rep: DataFrame containing the features.

- ind: Index of the feature of interest.

- rt_tol: Retention time tolerance.

- Output:

  - The function returns the indices of the selected candidate features that fall within the specified retention time bounds.

#### Function Steps

- Initialization:

  - Extracts the feature of interest (pf) from the DataFrame rep using the provided index ind.

  - Calculates the lower bound (lb) as the maximum of 0 and the feature's retention time (pf.Rt) minus the retention time tolerance (rt_tol).

  - Calculates the upper bound (ub) as the minimum of the maximum retention time in rep and the feature's retention time (pf.Rt) plus the retention time tolerance (rt_tol).

- Find Candidates:

  - Finds all entries in rep whose RT values fall within the bounds.

- Return:

  - Returns the indices of the selected candidates (sel_inds).

### 9. select_candidates_rt2_

This function selects candidate features for RT2 based on a specified retention time tolerance, ensuring that the selected features fall within the defined bounds.

#### Input and Output

- Input: The function processes a DataFrame of features, an index of the feature of interest, and a list of pre-selected indices.

- Required Columns in Input Files:

- rep: DataFrame containing the features.

- ind: Index of the feature of interest.

- sel_inds: List of pre-selected indices based on RT1.

- rt_tol: Retention time tolerance.

- Output:

  - The function returns the indices of the selected candidate features that fall within the specified retention time bounds for RT2.

#### Function Steps

- Initialization:

  - Extracts the feature of interest (pf) from the DataFrame rep using the provided index ind.

  - Calculates the lower bound (lb) as the maximum of 0 and the feature's RT2 (pf.Rt_2D) minus the retention time tolerance (rt_tol).

  - Calculates the upper bound (ub) as the minimum of the maximum RT2 in rep and the feature's RT2 (pf.Rt_2D) plus the retention time tolerance (rt_tol).

- Find Candidates:

  - Finds all entries in rep whose Rt_2D values fall within the bounds.

- Return:

  - Returns the indices of the selected candidates (sel_inds_f).

### 10. select_candidates_dot_

This function selects candidate features based on spectra similarity, ensuring that the selected features meet a specified similarity threshold.

#### Input and Output

- Input: The function processes a DataFrame of features, an index of the feature of interest, a list of pre-selected indices, a similarity threshold, and a mass tolerance.

- Required Columns in Input Files:

- rep: DataFrame containing the features.

- ind: Index of the feature of interest.

- sel_inds_f: List of pre-selected indices based on RT1 and RT2.

- sim_thresh: Spectral similarity threshold.

- mz_tol: Mass tolerance for matching m/z values.

- numfrags: Number of fragments in the spectra to be considered for comparison; passed from feature_align_external.

- similarity_method: Spectral similarity method: "DISCO" (DIstance & Spectrum Correlation Optimization) or "NDP" (Normalized Dot Product); passed from feature_align_external.

- Output:

  - The function returns the indices of the selected candidate features that meet the specified similarity threshold.

#### Function Steps

- Initialization:

  - Extracts the feature of interest (pf) from the DataFrame rep using the provided index ind.

  - Extracts the spectrum of the feature of interest (pfs) using the getSpec function.

  - Initializes an array vals to store similarity values for each candidate.

- Loop through Candidates:

  - For each candidate, it extracts the spectrum and generates master vectors (master_vect_f and reverse_vect_f) using master_mass_gen function for both directions (feature to candidate and candidate to feature).

  - Limits the number of fragments to consider based on the spectra to a maximum of 150.

  - Mean centers each vector by subtracting the mean intensity values.

  - Calculates the similarity between the spectra for both directions and takes the minimum similarity value.

  - Stores the similarity value in the vals array, setting negative similarity scores to zero.

- Filter Candidates:

  - Filters candidates based on the similarity threshold (sim_thresh).

- Return:

  - Returns the indices of the selected candidates (sel_inds_s) that meet the similarity threshold.

### 11. group_candidates_external

This function groups candidates based on their file IDs and calculates various statistics for each group, ensuring that the data is properly aggregated and summarized.

#### Input and Output

- Input: The function processes a DataFrame of features and a list of file indices.

- Required Columns in Input Files:

- reps: DataFrame containing the features.

- Ind_files: List of file indices.

- Output:

  - The function returns various statistics and arrays filled with processed values for each group.

#### Function Steps

- Initial Check:

  - If reps is empty, it returns a set of default values.

- Calculate Statistics:

  - Retention Time (Rt): Calculates the minimum, maximum, and average 1D retention times.

  - Retention Time  2D (Rt_2D): Calculates the minimum, maximum, and average 2D retention times.

  - Retention Index (RI): Calculates the minimum, maximum, and median retention indices.

  - Library Retention Index (RIlib): Calculates the minimum, maximum, and median library retention indices, handling missing values appropriately.

- Initialize Arrays:

  - Initializes arrays for various attributes (e.g., intensity, ID, SPEC, etc.) with zeros or "NA".

- Process Each File:

  - For each file in ind_files, it selects the corresponding rows from reps based on the file ID (F_ID).

  - If multiple candidates are found for a file, it aggregates their values (e.g., sums intensities, concatenates names and other attributes).

  - If only one candidate is found, it directly assigns its values to the corresponding arrays.

- Return Values:

  - Returns the calculated statistics and the initialized arrays filled with processed values:

  - MinRt, MaxRt, AveRt

  - MinMass, MaxMass, AveMass

  - MinRI, MaxRI, MedRI

  - MinRIlib, MaxRIlib, MedRIlib

  - Int_, ID_, SPEC_, FR_, CAS_, IN_, INtr_, SIM_, HIGH_, BASM_, ACTM_, FWHH_, PROB_, EXPI_, OBSI_, DRI_, CLS_, ORI_, QUAM_.

### 12. feature_align_fraction

This function aligns RT1 fractions (parts) by selecting candidate features based on retention times, similarity, and other criteria, ensuring that the data is properly aligned and grouped.

#### Input and Output

- Input: The function processes a DataFrame of features and various parameters for alignment.

- Required Columns in Input Files:

- rep: DataFrame containing the features.

- name: Name of the feature.

- ind_files: List of file indices.

- rt_tol: Retention time tolerance for RT1.

- rt2_tol: Retention time tolerance for RT2.

- rt2_tol_lowA: Lower retention time tolerance for RT2 when peak height is low.

- rt2_tol_highA: Higher retention time tolerance for RT2.

- rt_tol_lowA: Lower retention time tolerance for RT1 when peak height is low.

- h_thresh: Peak height threshold.

- rt_tol_highA: Higher retention time tolerance for RT1.

- sim_thresh: Similarity threshold.

- mz_tol: Mass tolerance for matching m/z values.

- highTol: Boolean indicating if high tolerance is used.

- feature, Inten, Idents, Specs, Forms, Cas, Inchi, InchiT, Sim, High, BasM, ActM, Fwhh, Prob, ExpI, ObsI, dRI, Cls, Ori, QuaM: Arrays to store the aligned features and their attributes.

- maxRTmatch: Maximum retention time for matching features.

- Output:

  - The function returns the aligned features and the remaining ungrouped features.

#### Function Steps

- Initialization:

  - Calculates the number of features outside the maximum retention time (maxRTmatch).

  - Initializes a loop counter (i) and an iteration counter (iteration_count).

- Loop through Features:

  - Iterates over each feature in the rep DataFrame.

  - Skips features with retention times of 0 or greater than maxRTmatch.

  - Adjusts the retention time tolerance if the peak height is below a threshold (h_thresh).

- Select Candidates:

  - Uses select_candidates_rt1_, select_candidates_rt2_, and select_candidates_dot_ to select candidate features based on retention times and similarity.

  - If candidates are found, it groups them using group_candidates_external and updates the rep DataFrame to mark the selected features as processed.

- Reduce DataFrame:

  - Periodically reduces the rep DataFrame to remove processed features and reset the loop counter.

- Return:

- Returns the aligned features and the remaining ungrouped features:

- feature, Inten, Idents, Specs, Forms, Cas, Inchi, InchiT, Sim, High, BasM, ActM, Fwhh, Prob, ExpI, ObsI, dRI, Cls, Ori, QuaM, rep_rem.

### 13. feature_align_external

This function performs a comprehensive alignment of features from multiple .csv files based on one of two methods ("DISCO" (DIstance & Spectrum Correlation Optimization) or "NDP" (Normalized Dot Product)), ensuring consistency and handling missing values appropriately. It processes the data to create aligned and cleaned DataFrames with various attributes.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- F_ID: Unique identifier for each entry.

- Name: Name of the entry.

- R.T. (s): Retention time in seconds.

- Area: Area under the curve.

- Spectrum: Spectrum data.

- Formula: Chemical formula.

- CAS: CAS number.

- InChIKey: InChIKey values.

- Retention Index: Retention index.

- Lib. RI: Library retention index.

- Height: Peak height.

- Similarity: Similarity score.

- Base Mass: Base mass.

- Actual Masses: Actual masses.

- FWHH (s): Full width at half height in seconds.

- Quant Masses: Quantitative masses.

- Probability: Probability values.

- Expected Ion m/z: Expected ion mass-to-charge ratio.

- Observed Ion m/z: Observed ion mass-to-charge ratio.

- Classifications: Class information.

- Origin: Chemical origin information; created by the filter_signal_external function.

- Output:

  - The function generates multiple .csv files containing aligned data for various attributes and an overview table.

#### Function Steps

- Initialization:

  - Calls report_import_external to import and clean the data from the specified directory (path2files).

  - Initializes various arrays and matrices to store feature data, intensities, identifiers, spectra, and other attributes.

  - Prints a message indicating the start of the alignment process with the current timestamp.

- Sorting and Tolerance Setup:

  - Sorts the rep DataFrame by the 'Height' column in descending order.

  - Sets initial retention time tolerances (rt_tol and rt2_tol) and a flag (highTol) to indicate high peak height.

- Partitioning Data:

  - Divides the data into parts based on equal distribution of retention times (tr1_part_lim).

- Aligning Features in Parts:

  - For each part, selects features within the specified retention time range.

  - Calls feature_align_fraction to align features within the part.

  - Updates the rep DataFrame to remove processed features and keep track of remaining features.

- Aligning Remaining Features:

  - Iterates over the remaining features in rep and aligns them using the same process as above.

  - Periodically reduces the rep DataFrame to remove processed features.

- Creating DataFrames:

  - Creates DataFrames for various attributes (e.g., intensities, identifiers, spectra) and renames columns based on file names.

  - Combines these DataFrames with the feature DataFrame (table1).

- Sorting and Numbering:

  - Sorts the combined DataFrames by average retention times.

  - Adds a 'Nr' column to number the features.

- Writing Output Files:

  - Writes the aligned data to multiple .csv files in the specified directory.

  - Calls finalOverviewTable to create an overview table and writes it to a .csv file.

- Completion:

- Prints completion messages with start and end times.

### 14. internal_feature_merging

This function merges features of the same chemical origin within a sample, such as the main peak and tailing peaks, ensuring that the data is properly aggregated and merged based on retention times and spectral similarity.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- F_ID: Unique identifier for each entry.

- Name: Name of the entry.

- R.T. (s): Retention time in seconds.

- Area: Area under the curve.

- Spectrum: Spectrum data.

- Formula: Chemical formula.

- CAS: CAS number.

- InChIKey: InChIKey values.

- Retention Index: Retention index.

- Lib. RI: Library retention index.

- Height: Peak height.

- Similarity: Similarity score.

- Base Mass: Base mass.

- Actual Masses: Actual masses.

- FWHH (s): Full width at half height in seconds.

- Quant Masses: Quantitative masses.

- Probability: Probability values.

- Expected Ion m/z: Expected ion mass-to-charge ratio.

- Observed Ion m/z: Observed ion mass-to-charge ratio.

- Classifications: Class information.

- Origin: Chemical origin information; created by the filter_signal_external function.

- Output:

  - The function generates updated.csv files with merged features and a separate file tracking the merged pairs.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Iterates through each file in the directory, skipping irrelevant files.

- Data Preparation:

  - Reads the .csv file into a DataFrame df.

  - Creates a DataFrame df_m to track merged features.

  - Copies the original file for backup if not already done.

  - Removes entries with missing 'Area' values and sorts the DataFrame by 'Height'.

- Feature Merging:

  - Extracts retention times (tr1, tr2) using extractRTs.

  - For each feature, finds other features within specified retention time deviations (tr1_dev, tr2_dev).

  - Checks spectral similarity using master_mass_gen and merges features if similarity exceeds a threshold (sim_thresh).

  - Updates the 'Area' of the parent feature and tracks merged features in df_m.

- Saving Results:

  - Saves the updated DataFrame and the merged pairs to new .csv files.

  - Prints completion messages with start and end times.

### 15. filter_signal_external

This function classifies features based on their base mass and retention time (RT1/RT2) region, ensuring that the data is properly filtered and classified according to specified criteria.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- F_ID: Unique identifier for each entry.

- Name: Name of the entry.

- R.T. (s): Retention time in seconds.

- Area: Area under the curve.

- Spectrum: Spectrum data.

- Base Mass: Base mass.

- Origin: Chemical origin information (if not present, it will be added).

- Output:

  - The function generates updated.csv files with classified features, marking their origin based on the specified filter type.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Checks if retention time ranges (tr1_range, tr2_range) are specified.

- Data Preparation:

  - Iterates through each file in the directory, skipping irrelevant files.

  - Reads the .csv file into a DataFrame df.

  - Adds an 'Origin' column if not present and copies the original file for backup.

- Feature Classification:

  - Extracts retention times (rt1, rt2) using extractRTs if needed.

  - Finds features with base masses close to the specified base mass within a tolerance (mz_tol).

  - For each candidate feature, checks if it falls within the specified retention time ranges and verifies the presence and intensity of specified m/z values.

  - Updates the 'Origin' column to the specified filter type (filtType) if all conditions are met.

- Saving Results:

  - Saves the updated DataFrame to the original file.

  - Prints completion messages with start and end times.

### 16. filter_classifications

This function filters out rows where the 'Classifications' column contains unwanted values such as "Bleed", "Toluene", or "DCM", ensuring that the data is properly cleaned, and relevant features are retained.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- Classifications: Column containing LECO ChromaTOF classification information.

- Output:

  - The function updates the .csv files by removing rows with unwanted classifications.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Iterates through each file in the directory, processing only .csv files.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Classifications' column exists.

  - Replaces missing values in the 'Classifications' column with "NA".

  - Filters out rows where 'Classifications' contains "Bleed", "Toluene", or "DCM".

  - Saves the filtered DataFrame back to the original file.

  - Prints completion messages with start and end times.

### 17. filter_names

This function filters out rows where the 'Name' column contains unwanted values, such as siloxanes and other bleed compounds, ensuring that the data is properly cleaned, and relevant features are retained.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- Name: Column containing the names of the features.

- Output:

  - The function updates the .csv files by removing rows with unwanted names.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Prepares a list of escaped keywords for regex safety.

  - Iterates through each file in the directory, processing only .csv files.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Name' column exists.

  - Replaces missing values in the 'Name' column with "NA" and strips leading/trailing spaces.

  - Filters out rows where 'Name' contains any of the specified keywords.

  - Saves the filtered DataFrame back to the original file.

  - Prints completion messages with start and end times.

### 18. filter_names_if_missing_mz

This function filters out rows where the 'Name' column contains specific keywords and the spectra do not contain specified m/z values, ensuring that the data is properly cleaned, and relevant features are retained.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- Name: Column containing the names of the features.

- Spectrum: Column containing the spectrum data.

- Output:

  - The function updates the .csv files by removing rows with unwanted names and missing specified m/z values.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Prepares a list of escaped filters for regex safety.

  - Iterates through each file in the directory, processing only .csv files.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Name' and 'Spectrum' columns exist.

  - Replaces missing values in the 'Name' column with "NA" and strips leading/trailing spaces.

  - Filters out rows where 'Name' contains specific keywords and the spectra do not contain specified m/z values.

  - Saves the filtered DataFrame back to the original file.

  - Prints completion messages with start and end times.

### 19. filter_names_mz_postalign

This function filters out rows after alignment based on specific name and m/z conditions, ensuring that the data is properly cleaned and relevant features are retained, similar to filter_names_if_missing_mz.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- Name: Column containing the names of the features.

- Spectrum_Consensus: Column containing the consensus spectrum data.

- Output:

  - The function updates the .csv files by removing rows with unwanted names and missing specified m/z values, saving the filtered data to new files with a "_Fltrd.csv" suffix.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Prepares a list of escaped filters for regex safety.

  - Iterates through each file in the directory, processing only files containing "overview.csv".

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Name' and 'Spectrum_Consensus' columns exist.

  - Replaces missing values in the 'Name' column with "NA" and strips leading/trailing spaces.

  - Prints the original number of rows for debugging.

  - Finds rows matching keywords for debugging and prints a preview of the matched rows.

  - Prints the names and m/z values found in the input files for debugging.

  - Filters out rows where 'Name' contains specific keywords and the spectra do not contain specified m/z values.

  - Prints the number of rows after filtering and the number of rows removed for debugging.

  - Saves the filtered DataFrame back to a new file with "_Fltrd.csv" suffix.

  - Prints completion messages with start and end times.

### 20. filter_peak_names_with_bleed_mz

This function filters out rows based on 'Name' containing "Peak" and specific bleed-related m/z values for the highest intensity pair in 'Spectrum', ensuring that the data is properly cleaned, and relevant features are retained.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- Name: Column containing the names of the features.

- Spectrum: Column containing the spectrum data.

- Output:

  - The function updates .csv files by removing rows with unwanted names (“Peak…”) and specific bleed-related m/z values for the highest intensity pair in 'Spectrum'.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Prepares a list of escaped filters for regex safety.

  - Iterates through each file in the directory, processing only .csv files.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Name' and 'Spectrum' columns exist.

  - Replaces missing values in the 'Name' column with "NA" and strips leading/trailing spaces.

  - Prints the original number of rows for debugging.

  - Finds rows matching keywords for debugging and prints a preview of the matched rows.

  - Prints the names and m/z values found in the input files for debugging.

  - Filters out rows where 'Name' contains "Peak" and the highest intensity m/z is one of the specified values.

  - Prints the number of rows after filtering and the number of rows removed for debugging.

  - Saves the filtered DataFrame back to the original file.

  - Prints completion messages with start and end times.

### 21. merge_postalign

This function merges features with the same name, base mass, and ΔRI < threshold after alignment, ensuring that the data is properly aggregated and merged based on specified criteria.

#### Input and Output

- Input: The function processes multiple .csv files in a specified directory.

- Required Columns in Input Files:

- Name: Column containing the names of the features.

- Base Mass: Base mass of the feature.

- MinRt1D, MaxRt1D, AveRt1D: Min, max, and average retention times for 1D.

- MinRt2D, MaxRt2D, AveRt2D: Min, max, and average retention times for 2D.

- MinRI, MaxRI, MedRI: Min, max, and median retention indices.

- MinRIlib, MaxRIlib, MedRIlib: Min, max, and median library retention indices.

- Area, TotalArea: Area values.

- Similarity: Similarity score.

- Probability: Probability values.

- dRI: Delta retention index.

- Score: Calculated score (based on specified weights).

- Spectrum_BestMatch, Spectrum_Consensus: Spectrum data.

- All_Names, All_Formulae, All_InChIKey, All_TotalArea, All_BaseMass, All_Origin, All_Class, All_CAS, All_Similarity: Aggregated information.

- DetectionNum, DetectionFreq: Detection frequency information.

- Output:

  - The function generates updated .csv files with merged features ("_Mrgd.csv" suffix) and a separate file tracking the initial features involved in the merge ("_Mrgd_Ftrs.csv" suffix).

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Iterates through each file in the directory, processing only files containing "overview_Fltrd.csv".

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Fills empty cells with 'NA'.

  - Deletes specified columns and sorts the DataFrame by 'Name'.

  - Iterates through the DataFrame, merging rows with the same name, base mass, and dRI < threshold.

  - Updates various columns (e.g., retention times, areas, spectra) during the merge.

  - Prints the initial and final number of rows for debugging.

  - Saves the merged DataFrame and the initial features involved in the merge to new files with "_Mrgd.csv" and "_Mrgd_Ftrs.csv" suffixes.

  - Prints completion messages with start and end times.

### 22. findMaxSim

This function retrieves the feature with the maximum similarity score from a list of similarity scores, probabilities, and retention index differences, ensuring that the best feature is identified based on specified criteria.

#### Input and Output

- Input: The function processes arrays of similarity scores, probabilities, retention index differences, and weights.

- Required Columns in Input Files:

- sim: Array of similarity scores.

- prob: Array of probabilities.

- dri: Array of retention index differences.

- w: Array of weights for calculating the score.

- Output:

  - The function returns the indices of the best feature (indc, inde), similarity value (val), probability (prob_), retention index difference (dri_), and score (score).

#### Function Steps

- Initialization:

  - Initializes variables to keep track of the best index (indc), sub-index (inde), similarity value (val), probability (prob_), retention index difference (dri_), and score (score).

- Loop through Similarity Scores:

  - Iterates over each similarity score in the sim array.

  - If the similarity score is not "NA", it checks if it contains multiple values (indicated by " &&& ").

  - If multiple values are present, it splits and parses them into arrays of floats.

  - Calculates the score for each value using the weights w and updates the best score if a higher score is found.

  - If only a single value is present, it parses the value and calculates the score directly.

- Return:

  - Returns the indices (indc, inde), similarity value (val), probability (prob_), retention index difference (dri_), and score (score).

#### Formula for Calculations

#### score= (valc⋅w[1]+probc⋅w[2]+dric⋅w[3])w

- where:

  - w: The weights assigned to each component (similarity (valc), probability (probc), and retention index difference (dric)) in the score calculation.

### 23. libraryVisualization

This function creates head-to-tail graphs for user vs. library spectra, ensuring that the data is visually represented for comparison.

#### Input and Output

- Input: The function processes a .csv file containing identification results and an index specifying which rows to process.

- Required Columns in Input Files:

- Spec: User spectrum data.

- DB_spec: Database spectrum data.

- Align_Num: Alignment number.

- DB_Num: Database number.

- Output:

  - The function generates head-to-tail graphs for user vs. library spectra and updates the .csv file with the file paths of the generated graphs.

#### Function Steps

- Initialization:

  - Prints a message indicating the start of graph creation and records the start time.

  - Reads the identification results from the specified file into a DataFrame df.

  - Removes rows where the 'Spec' column contains missing values.

- Graph Creation:

  - Creates a folder for the graphs if it doesn't already exist.

  - Iterates through each identification result.

  - Extracts the user and database spectra and normalizes their intensities.

  - Creates a bar plot with the user spectrum on the top and the database spectrum on the bottom.

  - Annotates the peak tops with m/z values.

  - Saves the graph to a file and updates the 'Location' column in the DataFrame with the file path.

- Output:

  - Saves the updated DataFrame to a new file with a "_NoNA.csv" suffix.

  - Prints completion messages with start and end times.

### 24. PubChemRetriever

This function retrieves InChIKey, SMILES, CAS#, DTXSID, Uses, and Use Classification from PubChem based on existing “InChIKey” and "Name" (if InChIKey does not exist) column values. It also updates consensus InChIKey and CAS# values based on additional conditions.

#### Input and Output Files

- Input Files:

  - The function processes files in the specified directory (path2files) that contain "_Mrgd.csv" in their names.

  - Required Columns in Input Files:

    - Name: The name of the chemical compound.

    - InChIKey: The InChIKey identifier (if available).

    - CAS: The CAS number (if available).

- Output Files:

  - The function saves the updated DataFrame to new files with a "_PubChem.csv" suffix.

Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Iterates through each file in the directory, processing only files containing "_Mrgd.csv".

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Adds empty columns for PubChem data (InChIKey_PubChem, SMILES_PubChem, CAS_PubChem, DTXSID_PubChem, Uses, Use_Classification, InChIKey_Consensus, CAS_Consensus).

  - For each name in the DataFrame, retrieves the corresponding CID from PubChem.

  - Uses the CID to retrieve InChIKey, SMILES, CAS#, DTXSID, Uses, and Use Classification values from PubChem.

  - Updates the DataFrame with the retrieved values.

- Copies all InChIKey_PubChem and CAS_PubChem values to InChIKey_Consensus and CAS_Consensus.

- Updates InChIKey_Consensus based on conditions: if "NA" in InChIKey_PubChem, copies from InChIKey.

- Updates CAS_Consensus based on conditions: if "NA" in CAS_PubChem, copies from CAS.

  - Sorts the DataFrame by InChIKey in descending order, with NAs at the bottom.

  - Saves the updated DataFrame to a new file with "_PubChem.csv" suffix.

### 25. ClassFinder

This function retrieves functional use data from EPA FUse, NORMAN dust trial, and in-house harmonized use databases. It integrates this data into an input DataFrame based on matching keys (Name, InChIKey_PubChem, and CAS_PubChem).

#### Input and Output Files

- Input File (path_file):

  - Required Columns:

    - Name: The name of the chemical compound.

    - InChIKey_PubChem: The InChIKey identifier from PubChem.

    - CAS_PubChem: The CAS number from PubChem.

- Harmonized Classification File (path_harm):

  - Required Columns:

    - Name: The name of the chemical compound.

    - InChIKey: The InChIKey identifier.

    - CAS: The CAS number.

    - Class: The harmonized classification.

- EPA FUse Classification File (path_fuse):

  - Required Columns:

    - Name: The name of the chemical compound.

    - InChIKey: The InChIKey identifier.

    - CAS: The CAS number.

    - harmonized_function: The functional use classification from EPA FUse.

- NORMAN Dust Trial Classification File (path_norm):

  - Required Columns:

    - Name: The name of the chemical compound.

    - InChIKey: The InChIKey identifier.

    - CAS: The CAS number.

    - Class: The classification from NORMAN dust trial.

- Output Files:

  - The function saves the updated DataFrame to a new file with a "_Cls.csv" suffix.

#### Function Steps

- Initialization:

  - Records the start time and prints a message indicating the start of the classification process.

  - Reads the input file (path_file) into a DataFrame (input_df).

  - Strips extra spaces from column names and adds empty classification columns.

  - Adds empty columns for the classifications (Class_Harmonized, Class_FUse, Class_NORMAN).

- Data Preparation:

  - Ensures the necessary columns (Name, InChIKey_Consensus, CAS_Consensus) exist in the input DataFrame.

  - Defines a helper function read_classification_file to read and process the classification files.

  - Reads the harmonized, EPA FUse, and NORMAN dust trial classification files into DataFrames (class_harm, class_fuse, class_norm).

  - Strips any extra spaces from column names and removes quotes from the Name column if present.

  - Ensures the necessary columns (Name, InChIKey, CAS) exist in the classification files. If not, automatically detects columns with similar names such as Feature name, feature_name, CAS#, CASRN, inchikey, etc. Renames columns in the classification files to match the input file's column names (InChIKey to InChIKey_Consensus, CAS to CAS_Consensus).

- Classification Update:

  - Defines a function update_class! to update the classification columns in the input DataFrame based on the classification files.

  - Updates the classification columns (Class_Harmonized, Class_FUse, Class_NORMAN) based on matching keys (Name, InChIKey_Consensus, CAS_Consensus).

- Finalization:

  - Fills empty cells with "NA".

  - Saves the output DataFrame to a new file with "_Cls.csv" suffix.

  - Prints a message indicating the successful completion of the classification process and the file path of the output file.

### 26. librarySearch

This function searches a library/database post-alignment to potentially identify unknowns and verify previous identifications. It uses retention index (RI) values and dot product calculations based on one of two methods ("DISCO" (DIstance & Spectrum Correlation Optimization) or "NDP" (Normalized Dot Product)) to find the best matches.

#### Input and Output Files

- Database File (pathDB); the database file should be an Excel file containing the library/database information:

  - Required Columns:

    - Columns with retention index (RI) values (our columns “RI”, “RI StdNP”, “RI StdPolar”, and “RI Est”).

    - Spectrum: The spectrum data for each entry in the database.

    - Name: The name of the chemical compound.

    - InChIKey: The InChIKey identifier.

    - CAS Number: The CAS number.

    - Formula: The chemical formula.

- Candidate Spectra File (pathFile); the file containing the candidate spectra to be identified:

  - Required Columns:

    - Nr: The number or identifier for each candidate spectrum.

    - MinRI: The minimum retention index value.

    - MaxRI: The maximum retention index value.

    - MinRIlib: The minimum library retention index value.

    - MaxRIlib: The maximum library retention index value.

    - Spectrum_BestMatch: The best match spectrum data.

    - Spectrum_Consensus: The consensus spectrum data.

    - Name: The name of the chemical compound.

    - BaseMass: The base mass of the compound.

    - Origin: The origin of the compound.

    - Class: The class of the compound.

    - All_Names: All names associated with the compound.

    - All_BaseMass: All base masses associated with the compound.

    - All_Origin: All origins associated with the compound.

    - All_Class: All classes associated with the compound.

- Output Files:

  - The function saves the identification results to new files with a suffix indicating the type of spectra used ("_LibSearch_BM.csv" or "_LibSearch_CS.csv").

#### Function Steps

- Initialization:

  - Prints a message indicating the start of the library search and records the start time.

  - Loads the database and overview files.

  - Creates retention index (RI) columns using the mergeToLeft function.

- Data Preparation:

  - Reads the candidate spectra from the specified file into a DataFrame df.

  - Replaces missing values in specific columns with "NA".

  - Determines which spectra to use based on the specID argument ("BM" for best match or "CS" for consensus spectra).

- Candidate Selection:

  - Iterates through each candidate spectrum.

  - Calculates the minimum and maximum RI values, considering library information.

  - Selects candidates from the database that fall within the RI range.

- Dot Product Calculation:

  - For each selected candidate, calculates the dot product between the candidate spectrum and the database spectrum using master_mass_gen. The calculations are based on the specified method: "DISCO" (DIstance & Spectrum Correlation Optimization) or "NDP" (Normalized Dot Product).

  - Keeps the top 3 matches based on the dot product.

- Output:

  - Saves the identification results to a new file with a suffix indicating the type of spectra used ("_LibSearch_BM.csv" or "_LibSearch_CS.csv").

### 27. mergeToLeft

This function merges RI values to the leftmost non-missing value and pairs them with corresponding RI tolerance values.

- Initialization:

  - Initializes arrays RIval and RItol with NaN values.

- Merging Values:

  - Iterates through each row of the input matrix ri.

  - For each row, assigns the leftmost non-missing RI value (out of four columns “RI”, “RI StdNP”, “RI StdPolar”, and “RI Est”) and the corresponding tolerance value from addRIwin.

- Output:

  - Converts RIval to a float array and replaces any remaining missing values with NaN.

  - Returns the merged RI values and tolerance values.

### 28. remove_zero_intensity_pairs

This function removes m/z:intensity pairs with intensity values containing '0.' from spectra in .csv files.

#### Input and Output Files

- Input Files; the function processes files in the specified directory (path2files) that are in .csv format:

  - Required Columns:

    - Spectrum: The spectrum data for each entry in the file.

- Output Files:

  - The function saves the filtered DataFrame back to the original file.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Iterates through each file in the directory, processing only .csv files.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Spectrum' column exists.

  - Replaces missing values in the 'Spectrum' column with "NA".

  - Removes m/z:intensity pairs with intensity values containing '0.' from the spectra.

  - Saves the filtered DataFrame back to the original file.

### 29. remove_low_intensity_pairs

This function removes m/z:intensity pairs with intensity values <= 3% of the highest intensity values for each spectrum in .csv files.

#### Input and Output Files

- Input Files; the function processes files in the specified directory (path2files) that are in .csv format:

  - Required Columns:

    - Spectrum: The spectrum data for each entry in the file.

- Output Files:

  - The function saves the filtered DataFrame back to the original file.

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Iterates through each file in the directory, processing only .csv files.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Checks if the 'Spectrum' column exists.

  - Replaces missing values in the 'Spectrum' column with "NA".

  - Removes m/z:intensity pairs with intensity values <= 3% of the highest intensity values from the spectra.

  - Saves the filtered DataFrame back to the original file.

### 30. escape_regex

This function escapes special regex characters in a string.

- Functionality:

  - Replaces special regex characters with their escaped versions.

  - Returns the escaped string.

### 31. calcTotalArea

This function calculates the total area for each entry based on the provided area, quant mass, and spectrum. It normalizes the area using the ratio of quant mass abundance to total abundance.

#### Input and Output

- Input Arrays:

  - area: An array containing the area values for each entry. Numeric values representing the area for each entry.

  - qmass: An array containing the quant mass values for each entry. Strings representing the quant mass for each entry, formatted as "QuantMass ±0.5".

  - spec: An array containing the spectrum data for each entry: Strings representing the spectrum data for each entry, formatted as "m/z:intensity".

- Output:

  - totArea: An array containing the calculated total area for each entry.

#### Function Steps

- Initialization:

  - Initializes an array totArea with zeros, having the same length as the input area array.

- Loop through Entries:

  - Extracts the spectrum using the getSpec function.

  - Parses the quant mass from the qmass string.

  - Finds the abundance of the quant mass in the spectrum.

  - Calculates the total abundance of all m/z values in the spectrum.

  - Normalizes the area using the ratio of quant mass abundance to total abundance.

  - Updates the totArea array with the calculated total area for the entry.

- Return:

  - Returns the calculated total area array totArea.

#### Equation for Calculations

#### Total Area= Area(Quant Mass AbundanceTotal Abundance)

- where:

  - Area: The area value for the entry.

  - Quant Mass Abundance: The abundance of the quant mass in the spectrum.

  - Total Abundance: The total abundance of all m/z values in the spectrum.

### 32. extractRTs

This function extracts RT1 and RT2 values from the 'R.T. (s)' column in the DataFrame Rep.

#### Input and Output

- Input:

  - The function processes a DataFrame (Rep) containing retention time data (required column R.T. (s): A string column containing retention time values formatted as "RT1, RT2").

- Output:

  - RT1: An array containing the extracted RT1 values.

  - RT2: An array containing the extracted RT2 values.

#### Function Steps

- Initialization:

  - Initializes a 2D array rts with zeros , having the same number of rows as the input DataFrame Rep and two columns for RT1 and RT2 values.

- Loop through Entries:

  - For each entry, it splits the 'R.T. (s)' string into RT1 and RT2.

  - Parses the split values into floating-point numbers.

  - Parses the split values into floating-point numbers.

- Return:

  - Returns the RT1 and RT2 arrays.

### 33. finalOverviewTable

This function creates an aligned overview table from various input tables, merging spectra and calculating detection frequencies and other statistics. It integrates information from multiple sources and provides a comprehensive summary of the data.

#### Input and Output

- Input; the function processes multiple input tables containing various attributes of the data.

  - Required Columns in Input Tables:

  - table_int: Contains intensity values and other initial columns.

  - table_id: Contains identification information.

  - table_sp: Contains spectrum data.

  - table_fr: Contains formula information.

  - table_cas: Contains CAS numbers.

  - table_in: Contains InChIKey values.

  - table_intr: Contains truncated InChIKey values (if used).

  - table_sim: Contains similarity scores.

  - table_high: Contains height values.

  - table_prob: Contains probability values.

  - table_dri: Contains delta RI values.

  - table_basm: Contains base mass values.

  - table_actm: Contains actual mass values.

  - table_cls: Contains class information.

  - table_ori: Contains chemical origin information; created by the filter_signal_external function.

  - table_qua: Contains quant mass values.

- Output:

  - overview: A DataFrame containing the comprehensive aligned overview table with all calculated and merged information.

  - Columns and Data:

  - Nr: Sequential number assigned to each feature. Data Type: Integer.

  - MinRt1D: Minimum retention time in the first dimension. Data Type: Float.

  - MaxRt1D: Maximum retention time in the first dimension. Data Type: Float.

  - AveRt1D: Average retention time in the first dimension. Data Type: Float.

  - MinRt2D: Minimum retention time in the second dimension. Data Type: Float.

  - MaxRt2D: Maximum retention time in the second dimension. Data Type: Float.

  - AveRt2D: Average retention time in the second dimension. Data Type: Float.

  - MinRI: Minimum retention index. Data Type: Float.

  - MaxRI: Maximum retention index. Data Type: Float.

  - MedRI: Median retention index. Data Type: Float.

  - MinRIlib: Minimum library retention index. Data Type: Float.

  - MaxRIlib: Maximum library retention index. Data Type: Float.

  - MedRIlib: Median library retention index. Data Type: Float.

  - DetectionNum: Number of detections. Data Type: Integer.

  - DetectionFreq: Detection frequency as a percentage. Data Type: Integer.

  - Name: Identifier for the feature. Data Type: String.

  - Formula: Chemical formula for the feature. Data Type: String.

  - InChIKey: InChIKey values for the feature. Data Type: String.

  - CAS: CAS number for the feature. Data Type: String.

  - Area: Area of the feature. Data Type: Float.

  - TotalArea: Total area of the feature. Data Type: Float.

  - QuantMasses: Quantitative masses for the feature. Data Type: String.

  - BaseMass: Base mass for the feature. Data Type: String.

  - ActualMasses: Actual masses for the feature. Data Type: String.

  - Origin: Chemical origin information for the feature created by the filter_signal_external function. Data Type: String.

  - Class: Classifications for the feature. Data Type: String.

  - Similarity: Similarity scores for the feature. Data Type: String.

  - Probability: Probability values for the feature. Data Type: String.

  - dRI: Delta retention index for the feature. Data Type: String.

  - Score: Calculated score based on similarity, probability, and delta retention index. Data Type: Float.

  - Spectrum_BestMatch: Best match spectrum data for the feature. Spectrum_BestMatch is the spectrum that most closely matches the feature's observed data (user spectrum) from a reference library or database. It represents the best alignment of m/z values and intensities between the feature and known spectra. Data Type: String.

  - All_Names: Identifiers for the feature. Data Type: String.

  - All_Formulae: Chemical formulas for the feature. Data Type: String.

  - All_InChIKey: InChIKey values for the feature. Data Type: String.

  - All_TotalArea: All total intensity values for the feature. Data Type: String.

  - All_QuantMasses: Quantitative masses for the feature. Data Type: String.

  - All_BaseMass: Base masses for the feature. Data Type: String.

  - All_ActualMass: Actual masses for the feature. Data Type: String.

  - All_Origin: Origin information for the feature. Data Type: String.

  - All_Class: Classifications for the feature. Data Type: String.

  - All_CAS: CAS numbers for the feature. Data Type: String.

  - All_Similarity: All similarity scores for the feature. Data Type: String.

  - Spectrum_Consensus: Consensus spectrum data for the feature. Spectrum_Consensus is a unified representation of spectral data for a feature, derived from multiple individual spectra that were aligned. It combines and normalizes the intensities of the same m/z values across different spectra to highlight the most consistent signals. Data Type: String.

  - N columns with total area for each of the aligned sample, where N is the number of samples aligned.

#### Function Steps

- Initialization:

  - Initializes the overview table with the first 13 columns of table_int.

- Calculate Detection Frequencies:

  - Calculates the number and frequency of detections for each entry.

- Initialize Columns:

  - Initializes columns for various attributes (e.g., Name, Formula, InChIKey, etc.) with "NA" or NaN.

- Fill in All Info:

  - For each entry, it fills in all information by concatenating unique values from the input tables.

- Consensus Spectrum:

  - Merges spectra for each entry based on the specified merging method (height-based or equal contribution).

- Highest Similarity Match:

  - Finds the highest similarity match for each entry and fills in the corresponding information.

- Calculate Total Areas:

  - Recalculates the total area for each entry.

- Return:

  - Returns the final overview table with all calculated and merged information.

#### Equations for Calculations

- DetectionNum

#### DetectionNum= i=1n(Areai>0)

- where:

  - n: The number of columns in the area table.

- DetectionFreq

#### DetectionFreq=(i=1n(Areai>0)n)×100%

- where:

  - n: The number of columns in the area table.

### 34. resort_delete_cols

This function reorders the columns in a .csv file according to a specified metadata columns order and deletes user-specified low-importance or QC columns.

#### Input and Output Files

- Input Files; the function processes a single .csv file specified by fileColDrop:

  - Required Columns:

    - None are strictly required, but column names should match those listed in meta_order for proper reordering.

  - Columns to Drop:

    - Specified via the col_names vector; only exact matches are removed.

- Output Files:

  - The processed DataFrame is saved as a new .csv file in the directory path2files with the specified suffix appended to the filename (default: _Final).

#### Function Steps

- Initialization:

  - Reads the directory contents and initializes a timestamp.

  - Checks whether the input file exists; raises an error if not.

- Data Processing:

  - Reads the .csv file into a DataFrame df.

  - Prints the original column names for reference.

  - Identifies which metadata columns from meta_order exist in the DataFrame.

  - Reorders df safely according to the final order of metadata and sample columns.

  - Prints the list of user-specified columns to drop.

  - Matches these names against the DataFrame columns and removes exact matches only.

  - Saves the filtered DataFrame back to the file originalFilename_Final.